North  South  University

Center of Excellence in Higher Education

CSE231.2

Term Project-SP18

Phase 2: Sequential Circuit design

Group : 7

Group Members:

| M.J. Darad | 162 0520 042 |
| --- | --- |
| Sajjad Hasan Rabbi | 151 2138 042 |
| Nusrat Jahan Khan | 151 2138 042 |
| Rubayet Zaman | 162 0684 642 |

Submitted to:

Dr. Arshad M. Chowdhury

Submission date:	23.03.2018

Objectives:

- To design the sequential logic part of the system

- Design a complete logic system from specification to implementation

- Coordination between the combinational and the sequential parts

- Become familiarized with the analysis of combinational logic networks and Synchronous Sequential Logic

- Learn the implementation of networks using Logic gates, decoders or MUX

- We need to display “SP18-CSE231-7” if the Direction input is logic LOW and the reverse order (“7-132ESC-81PS”) if the Direction input is logic HIGH

Phase 2: Sequential Circuit design

List of Equipment:

- IC 7473 (JK Flip-Flop) - 2 piece

- IC 7421 (4-input AND gate) - 1 piece

- IC 7411 (3-input AND gate) - 2 piece

- IC 7408 (2-input AND gate) - 2 piece

- IC 4072 (4-input OR gate) - 1 piece

- IC 4075 (3-input OR gate) - 1 piece

- IC 7432 (2-input OR gate) - 1 piece

- IC 7404 (NOT Gates) - 1 piece

- Voltage Regulator

- Voltage source

- Breadboard

- Switch

- Wires

State Diagram:

State Table:

| Input | Present State |  |  |  |  |  | Next State |  |  |  |  |  | Flip Flop Inputs |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | State | Bt | Ct | Dt | Et |  | State | Bt+1 | Ct+1 | Dt+1 | Et+1 |  | TB | TC | TD | TE |
| 0 | S | 0 | 0 | 0 | 0 |  | P | 0 | 0 | 0 | 1 |  | 0 | 0 | 0 | 1 |
| 0 | P | 0 | 0 | 0 | 1 |  | 1 | 0 | 0 | 1 | 0 |  | 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 0 | 1 | 0 |  | 8 | 0 | 0 | 1 | 1 |  | 0 | 0 | 0 | 1 |
| 0 | 8 | 0 | 0 | 1 | 1 |  | - | 0 | 1 | 0 | 0 |  | 0 | 1 | 1 | 1 |
| 0 | - | 0 | 1 | 0 | 0 |  | C | 0 | 1 | 0 | 1 |  | 0 | 0 | 0 | 1 |
| 0 | C | 0 | 1 | 0 | 1 |  | S | 0 | 1 | 1 | 0 |  | 0 | 0 | 1 | 1 |
| 0 | S | 0 | 1 | 1 | 0 |  | E | 0 | 1 | 1 | 1 |  | 0 | 0 | 0 | 1 |
| 0 | E | 0 | 1 | 1 | 1 |  | 2 | 1 | 0 | 0 | 0 |  | 1 | 1 | 1 | 1 |
| 0 | 2 | 1 | 0 | 0 | 0 |  | 3 | 1 | 0 | 0 | 1 |  | 0 | 0 | 0 | 1 |
| 0 | 3 | 1 | 0 | 0 | 1 |  | 1 | 1 | 0 | 1 | 0 |  | 0 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 | 0 |  | - | 1 | 0 | 1 | 1 |  | 0 | 0 | 0 | 1 |
| 0 | - | 1 | 0 | 1 | 1 |  | 7 | 1 | 1 | 0 | 0 |  | 0 | 1 | 1 | 1 |
| 0 | 7 | 1 | 1 | 0 | 0 |  | S | 0 | 0 | 0 | 0 |  | 1 | 1 | 0 | 0 |
| 0 |  | 1 | 1 | 0 | 1 |  |  | X | X | X | X |  | X | X | X | X |
| 0 |  | 1 | 1 | 1 | 0 |  |  | X | X | X | X |  | X | X | X | X |
| 0 |  | 1 | 1 | 1 | 1 |  |  | X | X | X | X |  | X | X | X | X |

| Input | Present State |  |  |  |  |  | Next State |  |  |  |  |  | Flip Flop Inputs |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | State | Bt | Ct | Dt | Et |  | State | Bt+1 | Ct+1 | Dt+1 | Et+1 |  | TB | TC | TD | TE |

| 1 | S | 0 | 0 | 0 | 0 |  | 7 | 1 | 1 | 0 | 0 |  | 1 | 1 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P | 0 | 0 | 0 | 1 |  | S | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 | 0 |  | P | 0 | 0 | 0 | 1 |  | 0 | 0 | 1 | 1 |
| 1 | 8 | 0 | 0 | 1 | 1 |  | 1 | 0 | 0 | 1 | 0 |  | 0 | 0 | 0 | 1 |
| 1 | - | 0 | 1 | 0 | 0 |  | 8 | 0 | 0 | 1 | 1 |  | 0 | 1 | 1 | 1 |
| 1 | C | 0 | 1 | 0 | 1 |  | - | 0 | 1 | 0 | 0 |  | 0 | 0 | 0 | 1 |
| 1 | S | 0 | 1 | 1 | 0 |  | C | 0 | 1 | 0 | 1 |  | 0 | 0 | 1 | 1 |
| 1 | E | 0 | 1 | 1 | 1 |  | S | 0 | 1 | 1 | 0 |  | 0 | 0 | 0 | 1 |
| 1 | 2 | 1 | 0 | 0 | 0 |  | E | 0 | 1 | 1 | 1 |  | 1 | 1 | 1 | 1 |
| 1 | 3 | 1 | 0 | 0 | 1 |  | 2 | 1 | 0 | 0 | 0 |  | 0 | 0 | 0 | 1 |
| 1 | 1 | 1 | 0 | 1 | 0 |  | 3 | 1 | 0 | 0 | 1 |  | 0 | 0 | 1 | 1 |
| 1 | - | 1 | 0 | 1 | 1 |  | 1 | 1 | 0 | 1 | 0 |  | 0 | 0 | 0 | 1 |
| 1 | 7 | 1 | 1 | 0 | 0 |  | - | 1 | 0 | 1 | 1 |  | 0 | 1 | 1 | 1 |
| 1 |  | 1 | 1 | 0 | 1 |  |  | X | X | X | X |  | X | X | X | X |
| 1 |  | 1 | 1 | 1 | 0 |  |  | X | X | X | X |  | X | X | X | X |
| 1 |  | 1 | 1 | 1 | 1 |  |  | X | X | X | X |  | X | X | X | X |

Karnaugh map (K-map):

TB =

TC =

TD =

TE =

Circuit Diagram :

Simulation :

Explanation:

Upon analysis of the table, it was clear that T Flip Flops would give a simplified circuit. However, T FFs were not available in the market so we decided to make T Flip Flops using JK Flip Flop. Behind this decision we had to consider a few facts for instance

efficiency, complexity, cost consideration etc.

From the figure we found that for both D Flip Flop and JK the Literal cost (L), Gate input cost (G), Gate input cost including inverters (GN) are much higher than the T Flip Flops. Also it will be a very complicated circuit where relatively more wiring is required and also the cost of implementation is higher. Thus we have decided to use T Flip Flops to implement the circuit.